SDR_AVE
Advanced Data Average Analyzer for SDR#
Based on the latest SDR# Plugin Template by Youssef Touil

Lead Developer: Daniel M. Kamiński
Chief Tester: Alex Pettit

Lublin (Poland), February 2026

# Introduction

SDR_AVE is a high-performance signal integration plugin for SDR#, ported to .NET 9. It features a slimmed-down, highly optimized codebase and a modernized graphical engine. For deployment, simply place SDRSharp.Average.dll in the Plugins folder; modern versions of SDR# will auto-detect the module.

# Control Panel: Button & Function Guide

- Enable Window: Activates the IF spectrum hook and opens the visualization window.

- Gain / Level Sliders: Control the vertical scale and position of the display (Logarithmic View).

- Signal Offset: Adds a constant bias (dB) to the display to calibrate power levels.

- Average Slider: Adjusts the integration constant for the second stage of averaging.

- Background Button: Captures the current average state as a reference to subtract noise.

- Reset Background: Clears the reference buffer and returns to normal magnitude mode.

- Single Save: Instant export of the current linear buffer to a text file.

- Multiple Save: Automated sequence recording with custom delays and file limits.

- Color Pickers: Persistent UI customization for text and graph glow (stored in Flags).

# Mathematical Engine: SDR_AVE Two-Stage Integration

The signal processing in SDR_AVE is split into two distinct integration stages to maximize performance and to eliminate the bias typical of logarithmic averaging while ensuring scientific accuracy.

## Stage 1: Intermediate Linear Integration

The software first accumulates N frames (where N=Intermediate_average) in the linear power domain. For each FFT bin i, the intermediate sum Si is calculated from the complex samples z:

Si=k=1NRezi,k2+Imzi,k2

Then, the linear average Plin,i is derived:

Plin,i=SiN

## Stage 2: Recursive Exponential Smoothing (The "Infinite Integrator")

To achieve high-ratio smoothing without consuming massive amounts of RAM, the second stage employs a Recursive Exponential Moving Average (REMA). This stage uses an alpha coefficient derived from the Average slider (M):

α=1M

The final smoothed linear value Sout,i is updated recursively:

Sout,i=1-α⋅Sprev,i+α⋅Pint,i

This method allows for extremely long integration times (up to 50,000 frames) with a memory footprint limited to a single FFT buffer.

The linear result undergoes further stabilizing the noise floor before conversion to the Decibel scale for the UI (this one is used only for graphical representation):

LdB,i=10⋅log10Plin,i+SignalOffset

## Total Integration Length

The total number of FFT frames contributing to the current state of the spectrum is the product of both stages:

Ntotal=Intermediate_average×Average

## Final UI Display

Only for visualization purposes, the data is converted to the Decibel scale:

DisplaydB,i=10⋅log10Sout,i+SignalOffset

# Data Storage Specification

A critical feature of SDR_AVE is the export of **Raw Linear Data**.

## Why Linear?

Data is saved **after the linear averaging stage but before the logarithmic conversion**. This preserves the physical integrity of the signal, allowing researchers to perform precise energy calculations in external software (Python/MATLAB) that would be impossible with "log-compressed" data.

## File Structure

All exports (Single, Multiple, and Background) use the following structure:

- Type: Plain Text (.txt).

- Format: A single row of space-separated floating-point values.

- Values: Represent Plin (Linear Magnitude) for each FFT bin.

- Example:  Frequency  |  Magnitude

0(100) – header (number of averaging samples)

86.126000000  0.000000210

86.135765625  0.000000228

86.145531250  0.000000238

86.155296875  0.000000300

86.165062500  0.000000293

- Convention: Invariant Culture (dot decimal separator).

# System Configuration

The plugin utilizes the Flags system to store settings in SDRSharp.Average.settings.txt. This includes window dimensions, DSP constants, and the ARGB values of your custom UI colors.

# Advanced Interference Mitigation: Median Filter (RFI Filter)

## Overview and Purpose

In amateur radio astronomy, the primary challenge is Radio Frequency Interference (RFI) from terrestrial sources. Standard linear averaging methods (arithmetic mean) are extremely sensitive to impulsive signals. A single high-energy pulse, such as lightning, automotive ignition, or radar sweeps, can severely bias the results of long-term integration.

The Median Filter implemented in version 3.3 introduces a non-linear pre-processing layer that eliminates outliers before the data accumulation stage in the EMA (Exponential Moving Average) buffer.

## Mathematical Foundation

The filter operates on a circular buffer of size N=5 frames for each frequency bin independently. For a given frequency f, let the set of power samples be P={p1,p2,p3,p4,p5}.

The filter output P is defined as the median of the set:

Pf=median{Pf}

When the samples are ordered in a non-decreasing sequence p1≤p2≤p3≤p4≤p5, the median corresponds to the third element of the sequence:

Pf=p3

Using a window length of 5 allows for the total rejection of up to two heavily corrupted frames in every measurement cycle without affecting the integrity of the underlying system noise floor.

## RFI Detection and Rejection Statistics

To enable data quality monitoring, the algorithm implements an RFI event detection system. A frame is classified as rejected (Xrej) if its instantaneous power at the center of the passband Pcurr exceeds the calculated median by a predefined threshold Γ:

Xrej⇔Pcurr>P⋅Γ

In the v3.3 implementation, the constant Γ is set to 10.0 (equivalent to a 10 dB power threshold). This statistic is presented in the GUI as the Rejection Rate:

R%=NrejectedNtotal⋅100%

## The Role of Sigma (σ)

The median filter serves as a robust estimator of the expected value, resistant to gross errors. In professional analysis, the deviation from the median allows for the determination of the σ-clipping threshold. A high R% value suggests that the statistical distribution of the noise is dominated by external factors rather than the system’s thermal noise (Gaussian distribution), serving as a warning for the observer.

## Operational Recommendations

- Data Integrity: The Median Filter should always be active when observing in urban or RFI-heavy environments.

- Integration Configuration: The highest efficiency in pulse detection is achieved with low Intermediate Averaging values (range 1–5).

- Interpreting Results: A steady increase in the REJECTED counter, even without visible changes on the live plot, indicates that the filter is successfully mitigating signals that would otherwise corrupt the averaged data.

# Advanced Visual Enhancement: Savitzky-Golay Smoothing

## Concept: Preserving Spectral Features

While standard temporal averaging (EMA) effectively reduces thermal noise over time, visual interpretation of a single spectrum often suffers from high-frequency "jitter." Conventional spatial smoothing (Moving Average) mitigates this but at the cost of spectral resolution, often resulting in the broadening and suppression of narrow emission lines, such as the Neutral Hydrogen (HI) line.

The Savitzky-Golay (S-G) filter is a digital polynomial filter designed to smooth data without significantly distorting the signal’s underlying features. It is particularly "sexy" for radio astronomy because it maintains the height and width of spectral peaks while flattening the surrounding noise floor.

## Mathematical Formulation

The Savitzky-Golay filter works by performing a local least-squares polynomial fit of degree k over a window of n points. For each frequency bin i, the smoothed value yi* is calculated as a weighted convolution of its neighbors:

yi*=j=-mmCj⋅yi+jj=-mmWj

Where:

- yi+j are the raw power values in the neighborhood of i.

- Cj are the Savitzky-Golay coefficients derived from the polynomial fit.

- 2m+1 is the window size (e.g., n=5 for m=2).

For a second-degree polynomial (k=2) and a 5-point window (n=5), the specific convolution coefficients C used in this plugin are:

C={-3,12,17,12,-3}, Normalization Factor=35

The negative weights at the edges of the window allow the filter to "track" the curvature of a peak much more accurately than a simple linear average.

## Scientific Integrity and Export Policy

A critical distinction is made in version 3.4 regarding data handling. To ensure scientific rigor, the Savitzky-Golay filter is applied exclusively to the visual output (GUI display).

- Visual Display: Optimized with S-G smoothing for easier feature identification and "publication-quality" real-time plots.

- Data Export (.txt): All exported files contain raw integrated linear power. This ensures that the statistical independence of frequency bins is preserved for professional post-processing and prevents the introduction of artificial correlations.

## Usage Recommendations

- High-Resolution Observation: Enable the S-G filter when searching for weak, narrow-band signals that might be obscured by rapid baseline fluctuations.

- Baseline Flattening: The filter is exceptionally effective at removing the "grassy" appearance of the noise floor, making the stable astronomical signal stand out.